Observer’s Checklist

Pilot’s Name		Date:

Before the Flight

Timing Watch

- Set watch using a GPS that is locked onto satellites.

GPS Flight Recorder (GPS-NAV)

- Install and seal the GPS-NAV to the glider.
Note: The seal is not required if the O.O. can keep the glider under continual observation until takeoff, and after landing until it is removed or the flight data is downloaded.

- Verify the following during the GPS-NAV power-up sequence:

- Displayed serial number should match the label on the GPS-NAV flight logger box.

- Geodetic Datum should read “100” which is WGS84.

Flight Declaration

- Make an electronic declaration in the GPS-NAV.

- Verify use of scratch distance for state (sports class) records

# Release

- Calculate maximum release altitude (3,281 feet for most tasks)

- Remember to do a tight 360° turn immediately after release (as a means of determining when free flight began).

## Relights

- Make a new electronic declaration in the GPS-NAV.
Note: Only one declaration is stored in the GPS-NAV so be sure to upload flight data between badge / record attempts.

During the Flight

Flight Data

- Record all times (take-off, tow-release, landing) on Observer’s Flight Data Form.

- Record takeoff and landing positions (as in "2,000 ft down Runway 18").

- Record tow-release location (i.e. ¼ mile SE of airport).

- Get tow-pilot and tow-plane information.

After the Flight

Post-Flight Data

- Check timing watch with official source (must be > 3 hours after it was set).

- All information should now be complete on the Observer’s Flight Data Form.

GPS Flight Recorder (GPS-NAV)

- Verify that GPS-NAV is still sealed to glider.

- Download flight log (both *.cai and *.igc files) onto 2 disks: one for the OO and one for the pilot.
Note: If the flight log cannot be downloaded immediately, the OO should remove the flight recorder and keep it in his/her possession until after the flight data has been downloaded in accordance with the IGC Approval Document.